Operation Instructions for Center Opening Door System

(TKP131-06/07)

NINGBO SHENLING LIFT ACCESSORIES CO., LTD.

# Contents

1. Products introduction………………………………………………………..1

2. Installation and adjustment…………………………………….…………...2

2.1 Installation of car door device………………………………………….2

2.2 Adjustment of car door device…………………………………………2

2.3 Installation of landing door system……………………………………5

2.4 Adjustment of landing door device ……………………………………6

2.5 Damageable parts…………………………………………………………7

3. Maintenance and protection…………………………………………………8

1  Products introduction

  - Technical parameter

Input voltage :AC220V±10% ; Rated output power of inverter : 400W ;

Door opening width :JJ700—1200mm .

  - Work condition

Height above sea level cannot exceed 1000m.

Temperature : -5℃－50℃;

Relative humidity can not exceed 35%(when temperature is 25℃);

Voltage:200V－250V, 50—60Hz

No explosion risk, no caustic or insulation destroys gas and electric dust.

1.3  Structure principle

TKP131-06 type adopts synchronous belt to drive non-synchronous door operator, VF control with non-synchronous door vane.

TKP131-07 type adopts synchronous belt to drive synchronous door operator, VF control with synchronous door vane.

Electric control system of above-mentioned two door operators are same, which adopts PLC to control inverter to vary speed, in order to have the best speed curve during opening and closing. It has main advantages as high efficiency, high reliability, high power factor and excellent variable speed, simple to operator, small starting current, less mechanical vibration, perfect protection and easy to closed loop adjustment and so on.

The main distinction of this two door operator is at structure: TKP131-07 adopts synchronous door vane, its mechanical device makes sure that draw in door vane first, deflate landing door header, and lead landing door to open, then let loose door vane, secede pulley of landing door header, during the whole opening and closing course, landing door keeps flat with car door. TKP131-06 is adopting non-synchronous door vane.

All of this two products decelerate by V-belt, drive by synchronous belt to run smoothly.

2  Installation and adjustment

2.1  Installation of car door device

2.1.1  Installation arm: Setting installation arm on the straight beam of cabin as per drawing 1, fixed temporarily after connected diagonal rod, and then tighten bolt after adjusted door operator; Install door operator with installation arm at A-point position and make sure the center of door operator coincide with the car entrance.

Drawing 1

2.1.2  Car door operator: Adjust the distance of door operator through the hole of installation arm as per drawing 1, make sure the horizontal distance is 58±1mm from rail to sill, and height is HH+87±1mm showed in drawing 2, connect horizontal drag link according to the general installation drawing. Above-mentioned dimension should be coincident at both sides of door operator. Height and horizontal distance between landing door rail and sill is HH+70 and 43.5 separately.

2.1.3  Car door panel: Fix the door panel by M8 bolt of small parcel as per drawing 3 and door shoe as per drawing 4, can increase or decrease washer between panel and hanger plate to adjust vertical if the panel is inclined. Insure the inclination and center gap of panel are at 2mm. Please refer to drawing 18.

Drawing 2                 Drawing 3                    Drawing 4

2.2  Adjustment of car door device

Products have been adjusted before came out factory, but considering that installation and movement may have a influence on precision of size, the followings need for adjusting on the spot:

2.2.1  Dimension from door vane protrude car sill: check the protrusive amount of door vane as per drawing 6, adjust the washer between panel and door vane if it is not measure up to requirement,

Drawing 5                         Drawing 6

2.2.2  Adjustment of door vane: Adjustment of door vane of TKP131-06 operator according to drawing 5, remain safe-locking when the door fully open, vane and roller adjusted according to the drawing 6(b)（A=B=10±5）; The position of door vane and roller TKP131-07 adjusted as per drawing 6(b)( A=6±1,B=13±1), adjustment of synchronous door vane of TKP131-07 door operator as per drawing 7: The following drawing shows closing status, the distance between two hanger plants is 50mm,when the door opening, lock device spring on right lock the cam, at the same time, moveable vane closed and 46mm instead of 72mm, then synchronous door vane and hanger plate move synchronously, door begin to open. Move lock cam towards or deflate lock device spring can amplify locking power.

2.2.3  Measure of door closing: the distance between two hanger plants should be 50mm when the door closes completely, please adjusts the protrusive length of hanger plate in few if not. Check whether the measure of door opening course is of attainment (TKP131-06 is JJ+32, TKP131-07 is JJ), please adjusts the protrusive of hanger plate few till ok if not.

2.2.4  Wire rope: Adjusted as per drawing 8

2.2.5  Synchronous belt tension: Measure and adjust the tension of synchronous belt following drawing 9, through moving the roller on left to accomplish decorous tension, in order to dispel noise which begot by friction between synchronous belt and roller.

2.2.6  Belt tension:

2.2.6.1  Adjust centerline of motor pulley and V-belt pulley to a plane, in order to dispel noise refers to drawing 10.

2.2.6.2  Measure and adjust tension of V-belt of motor following drawing 11, through moving the motor pedestal bolt to accomplish decorous tension.

2.2.7  The drag of hanger plate: Please adjust the interval of roller and rail under the hanger plate to 0.1-0.3mm can make door running smoothly if it gets too many drag.

Drawing 8                         Drawing 9

Drawing 10

Drawing  11                         Drawing  12

2.2.8  safety edge: The detailed adjustment of safety edge follows drawing 13(this drawing takes the right safety edge of TKP131-06 for instance).

2.2.8.1  Have been installed safety shoe on car door, adjust stopper , set the maximum value of shoe rotary bulge to 48±2mm;

2.2.8.2  Choose the length of connection rod A1 in accordance with required size and fix it temporary, then push the edge 16±2mm to the side of panel in full closed position, twist edge clockwise(contra clockwise if it is left), fix the cam and connection rod temporarily by 3 M6 bolts at the position that stopper touched hood, set the edge to ok.

Drawing  13

Drawing  14

2.2.8.3  Please confirm there is 13±2mm from the front side of safety edge in door fully opening position, should be decrease the length of connect rod (A1) if greater than 15mm, otherwise, increase the length.

2.2.8.4  When the door is fully open and the edge draws back at its maximal position, the stopper B is adjusted to touch to the arm. Then, the spring pin is inserted into and drill Ǿ4 holes and fixes it.

2.2.8.5  The micro-switch should be work when adjust the course of safety edge to 3-6mm with bolts.

2.3  Installation of landing door system

2.3.1  Sill support channel and sill

Use M12 hilti bolt to connect sill supporting channel with land, to keep center of sill and cabin entrance in the same position, and then use M8×20 bolt to fix sill to the supporting channel.

2.3.2  Jamb

Install according to the following drawing, to keep the center of jamb and cabin entrance in the same position, to ensure the door cover in the vertical position and fix it, and then solder jamb with pre-installed steel bars, and clean up to prevent rusty.

Drawing  15                      Drawing  16

2.3.3 Landing door device

Install the landing door device after finished the jamb, installation of landing door device and jamb according to drawing 16. Fix the landing door device temporarily with M2×100 hilti bolt in holes of the tope through installation stand. Tighten the bolts and nuts after adjusted over sling.

  - Adjustment of landing door device

2.4.1 confirm the center and adjust the plume line of over sling. Adjust the center and plume line of over sling as per drawing 17.

2.4.2 Installation and adjustment of landing door panel: Adjust the door hanger plate first as per drawing 19, then install and adjust landing door panel as per drawing 18. Measure the gap between up & down sides of panel and jamb in order to meet the requirement showed in drawing 18, and insure the plume line of door panel.

Drawing  17

2.4.3 Weight hammer: Connecting its top part to the side door, and bottom part to the panel seal. Then, put the weight hammer in the guide set, and hang a steel rope in pulley with another end fixed on the fixing board, use bolt to prevent any fall down, and keep the distance with pulley less than 2.5 mm and without touch.

Drawing  18

2.4.3 Drive wire rope

Wire rope tension of landing door device shall be adjusted by synchronous wire rope of door operator when the door fully opening, then, fixed by nuts after tighten.

2.4.5 Door lock

Adjust door lock as the dimension showed in drawing 20

Drawing  19                           Drawing  20

2.5  Damageable parts

Damageable parts for landing door device

| POS | Name of parts | Drawing | Qty/landing door |
| --- | --- | --- | --- |
| 01 | Special nut | X04RXG08 | 8 |
| 02 | Door slider | TA098-118G01 | 4 |
| 03 | Lock contact | 3UF0315 | 1 |
| 04 | Coupling | TA022-630G01 | 2 |
| 05 | Contact plate | SLB341736 | 1 |
| 06 | Insulation board | DTP16101-08-2 | 1 |
| 07 | Pressed plate | TA046-863 | 1 |
| 08 | Contact assy | TA046-858G01 | 1 |

Damageable parts for car door operator

| POS | Name of parts | Drawing | Qty/landing door |
| --- | --- | --- | --- |
| 01 | Special nut | X04RXG08 | 8 |
| 02 | Door slider | TA114-612G01 | 4 |
| 03 | V-Belt | L=1250 | 1 |

- Maintenance and protection

  - Motor bearing of the door should be added lithium lubricant, once a year.

  - Wipe rail when elevator is under maintenance, and check the deformation of hanger plant

roller, change it in time if deformation goes beyond 1.5mm.

  - When elevator is under maintenance, belt, wire rope and synchronous belt should be

adjusted in accordance with mentioned proceeding.

  - Common maintenance should check tension of standard parts.

  - Please check and change the rubber stopper of weight hammer if there is a unwonted

noise after use.

3.6  Regarding electrical adjustment please refers to operation manual of inverter.