Monochrome LCD position indicator CM300 CM540

The CM300, CM540 monochrome LCD display is a good value; innovative and visually attractive lift indicator display that was developed to provide a high-tech looking alternative to normal dot matrix displays.

The CM300, CM540 comes in two color combinations, eye-catching Green on yellow, and contrasting dark gray on white.

The display comes in two orientations, vertical and horizontal, each with their own advantageous features.

# Features

• Slim line, Low Profile, Small Size and Compact.

• Able to show lift positions, directional arrows

• Bright Green on yellow or contrasting dark gray on white

• Configuration of floor legends and arrow can be programmed into an EEPROM

• Can be horizontally or vertically mounted

# Applications

• Use as car position indicator on car operating panel

• Hall position indicator on hall landings

| ### Display Specification |  |
| --- | --- |
| Interface | One wire serial |
| Display Type | Monochrome STN LCD |
| Effective Viewing Area (mm) | 66 (W) x 33 (H) |
| Physical Product Size (mm) | 70 (W) x 140 (H) x 16 (D) |
| Resolution (pixels) | 128 x 64 |
| Dot Size | 0.48 x 0.48 mm |
| Dot Pitch | 0.52 x 0.52 mm |
| #### Viewing Angle X-axis For Landscape | -30°to +30° |
| Viewing Angle Y-axis For Landscape | 5°to +30° |
| Floor Legend and Arrow Height (variable) | Typically 40mm |
| Character Height for Messages (variable) | Typically 7mm |
| Absolute Maximum Ratings |  |
| Supply Voltage, DC | 12V to 30V |
| Supply Voltage, AC | 10V to 27V |
| Supply Current @ 12V DC | 0.3A |
| Supply Current @ 24V DC | 0.2A |
| Signal Voltage, DC (opto isolated input type) | 10V to 30V |
| Signal Current, DC (opto isolated input type) | <10mA |
| Operating Free-air Temperature Range | 0℃——50℃ |
| Storage Temperature Range | -20℃——70℃ |
| Humidity | 10% to 90% non condensing |

# Features

• Slim line, Low Profile, Small Size and Compact.

• Able to show lift positions, directional arrows, date , time and simple messages

• Bright Green on yellow or contrasting dark gray on white

• Configuration of floor legends and arrow can be programmed into an EEPROM

• Can be horizontally or vertically mounted

# Applications

• Use as car position indicator on car operating panel

• Hall position indicator on hall landings

| ### Display Specification |  |
| --- | --- |
| Interface | 12 inputs/RS485, RS232/7-SEG, Binary, BCD, Gray code et. |
| Display Type | Monochrome STN LCD |
| Effective Viewing Area (mm) | 108 (W) x 58 (H) |
| Physical Product Size (mm) | 144 (W) x 130 (H) |
| Resolution (pixels) | 240 x 128 |
| Dot Size | 0.4 x 0.4 mm |
| Dot Pitch | 0.45 x 0.45 mm |
| #### Viewing Angle X-axis For Landscape | -30°to +10° |
| Viewing Angle Y-axis For Landscape | 10°to +40° |
| Floor Legend and Arrow Height (variable) | Typically 36mm |
| Character Height for Messages (variable) | Typically 7mm |
| Absolute Maximum Ratings |  |
| Supply Voltage, DC | 10V to 30V |
| Supply Voltage, AC | 10V to 27V |
| Supply Current @ 12V DC | 0.5A |
| Supply Current @ 24V DC | 0.3A |
| Signal Voltage, DC (opto isolated input type) | 10V to 30V |
| Signal Current, DC (opto isolated input type) | <10mA |
| Operating Free-air Temperature Range | 0℃——50℃ |
| Storage Temperature Range | -20℃——70℃ |
| Humidity | 10% to 90% non condensing |

Full Color LCD position indicator CM840 (developing)

The CM840 full color displays not only provide standard information like position, directional arrow, date, time, and messages, but also time-based pictures and messages.

Using active matrix TFT, the displays provide a high-resolution effect. They are available in 8.4”, 10.4” screen sizes.