Table of Contents

- Special Precautions ………………………..............................................................1

- Terminal Function  …………………………………………..………….....................1

- Keypad &Operation ...…………………………………………….……………………,.2

- Connection Diagrams……………..……………………………………………………..3

- Door Run Description ………………………………………………………………….4

- Basic  procedure 1………………………………...………………...………………….6

- Basic  procedure 2……………………………………..………………………………..6

- Function Parameter Description …………………………………..…………………..9

9.  Troubleshooting  ………………….………...…………………..……........................13

1、Special Precautions

. Do not open or remove the front cover while the Variable

Frequency Drive is running. You may get an electrical shock.

. When necessary to perform inspections or when wiring the

unit, switch power off and wait at least  3  minutes and until

the bus charge light is off. Check for residual voltage.

. Do not attempt to inspect or wire this unit unless fully competent

to perform the work.

. Be sure hands are dry before operating any switches.

. Be sure cables do not have scratches, excessive stress,

. Be sure all connections are in accordance with instructions

in this manual

. Check that cables are properly connected before turning

equipment on.

. Do not allow metal fragments, conductive bodies, oil or other

flammable substance to enter the variable frequency drive.

. Do not modify this equipment

2、Terminal Function

| Name | Cable code （color ） | Function description |
| --- | --- | --- |
| L / N / PE | Brown /blue /(yellow&green) | Single phase power supply |
| U / V / W | U/V/W* | Motor terminal |
| 1 | 2# | Input signal: open door （NO） |
| 2 | 3# | Input signal: close door（NO） |
| 3 | --* | Input signal: open limit（NC） |
| 4 | --* | Input signal: close limit（NC） |
| 8 | 1# | Input signal commonality（COM） |
| C2-C3 | 4# | Limit output commonality |
| B2 | 5# | Output signal: open limit（NC） |
| B3 | 6# | Output signal: close limit （NC） |
| C1 | standby | Error output commonality（COM） |
| B1 |  | \Error output signal :break contact（NC） |
| A1 |  | Error output signal: make contact（NO） |

Notice : the cable with sign *has been connected in door system , the 7 wires control cable should be connected to main control system by user

3、Keypad &Operation

3、1 Keypad Definition

| Name | Function |
| --- | --- |
|  | To switch Mode:dr 、fr & parameter type : p、n |
|  | Go down one level in menu navigation ;confirm values after editing |
|  | scroll up numeric values & parameter |
|  | scroll down numeric values & parameter |
|  | Start key commands the drive to enable and start |
|  | Stop key commands to stop and disable ,or reset error |

3、2 Keypad Basic Operation

Setting example ：Change parameter P05 from 20 to 22

Operation                             LED display

Power supply

Press   
   to get relevant

parameter  type & relevant mode

Scroll press   
  &

to get relevant parameter

Press  
  to edit the parameter

Or  to get monitor

Scroll press   
 &

to  modify the parameter

Press 
  to confirm the parameter

Press   
   to  return

to operation  preparation

Notice ：parameter type d is behind parameter type p .  when get parameter type p, continue to press  button , can get parameter d.

4、Connection Diagrams

figure 1

Notice :

1. Make control line and power line separately when connect to main controller.

2. To be sure the match of the open/close door output logic of door controller and the Open/close door input signal logic of main controller, when connect to the signal of Open/close.

The initial status of door controller is normal close(NC).

User can select relevant logic status:

Terminal A2—open limit, normal open(NO).

Terminal A3—close limit, normal open(NO).

Terminal (C2-C3)---limit output, signal common terminal.

5、Door Run Description

5、1 During(Course of) Open Door

1、From DC torque boost level value P05、acceleration time d28 ,accelerate to start open frequency P15, then start with target frequency d16 and acceleration time d29 to perform open door low-speed running.

2 when running  time d47, start with acceleration time d30 running to high-speed target frequency d17, high-speed frequency be set a common section,d17=d19.

.

3、when door arrive position of open shift switch, after delay time d48,then start with deceleration time d33 to decelerate to open door low-speed target frequency d20.

4、when door arrive position of open door target position, start with keeping frequency d21,

Keeping current d40,to keep torque of door in that position.

Figure 2

notice：

- high-speed section of open usually is the same section as: d17=d19,  at this point the acceleration/ deceleration parameter: d32 is void.

2.  To keep frequency of door opening completed is d21 and the current is d40.

Keep time is d42,and the range is  0.1 to 999(s) ,when d42 is set  as 0,it will

be keep forever .

3、If opening torque of low-speed is small, the value of P05 can be increased

appropriately.but more current course system protect

5、2  Course of close Door

1、From torque boost level value P05、acceleration time d34 ,accelerate to low -speed open frequency d22, to perform original close door running. Then start with target frequency d23 and acceleration time d35, to perform low-speed close door running

2 when running  time d49, start with acceleration time d36 running to high-speed target frequency d24, high-speed frequency be set a common section,d24=d26

.

3、when door arrive position of open shift switch, after delay time d50,then start with deceleration time d39 to decelerate to open door low-speed target frequency d27.

4、when door arrive position of open door target position, start with keeping frequency d14,

Keeping current d41to keep torque of door in that position.

Figure 3

- Notice ：high-speed section of open usually is the same section as: d24=d26,  at this point the acceleration/ deceleration parameter: d38 is void.

2.  To keep frequency of door opening completed is d14 and the current is d41.

Keep time is d42,and the range is  0.1 to 999(s) ,when d42 is set  as 0,it will

be keep forever .

3、If opening torque of low-speed is small, the value of P05 can be increased

appropriately. but more current course system protect

6、Basic  procedure 1

Before the door controller is sent out from factory, the Autotune of door width pulses and function parameters are completed. If user requires getting preferable running curve, he can modify the corresponding parameters refer to fig.2 & fig. 3.

Procedure is:

- To set control mode is keypad control: p08=1.

- Refer to fig.2 & fig.3 and corresponding parameters (page:4,5), to get better capability through adjusting door running curve.

- Press  +RUN or +RUN (first press button and then press RUN button，following operation is the same ) to examine the running curve.

- Set control mode become terminal control: P08=2( default )

NOTICE: if the controller is running, P08 can not be changed. first please press “STOP” key, then press “MODE” key switch to parameter “P” and find P08 to modify

7、 Basic  procedure 2

When the motor shift switch or logic of limit switch(NO/NC) is changed,

Or autotune the door width pulses is done again. The following procedure must to be sure.

7.1. Confirmation of the I/O signal (parameter n16,n17).monitor.

Parameter n16 is the monitor of input signal, it includes: ①~⑨。

Parameter n17 is the monitor of output signal, it includes: Open limit output signal, close limit output signal, error output signal.

①--open door signal(input)/ RY1  OUTPUT

②--close door signal(input)/ RY2  OUTPUT

③--open limit signal(input)/ RY3   OUTPUT

④--close limit signal(input )

⑤--safety edge signal (input)

⑥-- open switch speed signal(input)                  .

⑦-- close switch speed signal(input)                           figure 4

notice: the terminal⑤,⑧,⑨is not defined,

So monitor signal⑤,⑧,⑨ is not work

Press  button to switch the parameter n mode ,and  pressor button To get relevant monitor parameter, then press   ,  can know the state of  signal refer to figure 4 .

7、2  Confirmation of the logic of input signal（NO/NC）

The logic of input signal parameter---P43

1、setting value:0----the signal terminal connect to common terminal of input signal ,the signal is enable（NO）.

1----the signal terminal disconnect to common terminal of input signal, the signal is enable（NC）.

2、setting way：Input of the decimal data of 0 to 15 bit.

Setting valueweights=total （as the setting value）

For example: the open limit input signal and the close limit input signal are set as NC, and others are set as NO.

So the setting value P43=016+08+04+12+11=3

| Signal | （NULL） | Open change signal | Close change signal | Safety edge signal | Open limit signal | Close limit signal |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal | ------ | 7 | 6 | 5 | 4 | 3 |
| power（BIT ） | 5—15 | 4 | 3 | 2 | 1 | 0 |
| value | 0 | 0/1 | 0/1 | 0/1 | 0/1 | 0/1 |
| Total | 0 | 16 | 8 | 4 | 2 | 1 |

7.3  affirm of the motor rotation direction

condition：The frequency is set as about 3Hz(in the mode Fr), and P09=0

process：press +RUN button，to sure open of the door；

press+RUN button， to sure close of the door.

notice：1. If motor dose not run, to increase properly the value of parameter P05.

2. If the rotation direction is not right, to change motor phases.

7、4control mode must to be sure

| The value of P08 | Mode |
| --- | --- |
| 0 | Keypad mode（RUN/STOP） |
| 1 | Keypad mode（+RUN—open, +RUN—close, STOP） |
| 2 | Terminal mode（NO：1--open、2--close） |
| 3 | Communication (RS 485) |

7、5running mode must to be sure

| The value of P09 | Control mode |
| --- | --- |
| 0 | manual(the frequency be set in the Fr mode ) |
| 1 | Encoder mode |
| 2 | Encoder circle mode |
| 3 | Door width pulse autotune |
| 4 | 4 position switch mode |
| 5 | 4 position switch circle mode |
| 6 | Communication mode |

8、Function Parameter Description

| NO | Name | Default | Range | Multiple | Unit | edit |
| --- | --- | --- | --- | --- | --- | --- |
| P00 | CHECK MODE | 0 | 0-3 | 1 | - | R |
| P01 | 1st ACCELERATION TIME | 0.5 | 0-999 | 10 | s | R/W |
| P02 | 1st DECELERATION TIME | 0.5 | 0-999 | 10 | s | R/W |
| P03 | FREQUENCY RANGE | 0 | 0-2 | 1 | s | R |
| P04 | V/F CURVE | 0 | 0-1 | 1 | Code | R |
| P05 | DC BOOST LEVEL | 15 | 0-40 | 1 | - | R/W |
| P06 | OVERLOAD FUNCTION | 2 | 0-3 | 1 | % | R |
| P07 | OVERLOAD CURRENT | 3.6 | 0.1-100 | 10 | - | R |
| P08 | CONTROL MODE | 2 | 0-3 | 1 | A | R |
| P09 | RUNNING MODE | 4 | 0-6 | 1 | - | R |
| P10 | STOP MODE SELECT | 0 | 0-1 | 1 | - | R |
| P11 | STOP FREQUENCY | 0.5 | 0.50-60 | 100 | - | R |
| P12 | DC BRAKE TIME | 0.5 | 0-120 | 10 | Hz | R |
| P13 | DCBRAKE LEVEL | 0.0 | 0-100 | 0.2 | s | R |
| P14 | MAX. FREQUENCY | 50.00 | 50-250 | 100 | - | R |
| P15 | BASE FREQUENCY | 50.00 | 45-250 | 100 | Hz | R |
| P16 | PREVENT OVERCURRENT STALL | 1 | 0-1 | 1 | Hz | R |
| P17 | PREVENT OVERVOLTAGEG STALL | 1 | 0-1 | 1 | Hz | R |
| P18 | SKIP FREQUENCY 1 | 0.00 | 0,0.5-250 | 100 | Hz | R |
| P19 | SKIP FREQUENCY 2 | 0.00 | 0,0.5-250 | 100 | s | R |
| P20 | SKIP FREQUENCY 3 | 0.00 | 0,0.5-250 | 100 | - | R |
| P21 | SKIP FREQUENCY BAND WIDTH | 0 | 0-10 | 1 | - | R |
| P22 | CURRENT LIMIT FUNCTION | 0.0 | 0-9.9 | 10 | s | R |
| P23 | POWER LOSS START MODE | 0 | 0-3 | 1 | - | R |
| P24 | RIDE-THROUGH RESTART | 0 | 0-2 | 1 | - | R |
| P25 | WAIT TIME | 0.1 | 0-0.1-1002 | 10 | s | R |
| P26 | SELECT RETRY | 0 | 0-3 | 1 | - | R |
| P27 | RETRY TIMES | 1 | 1-10 | 1 | Times | R |
| P28 | LOWER FREQUENCY CLAMP | 0.5 | 0.5-250 | 100 | Hz | R |
| P29 | UPPER FREQUENCY CLAMP | 250 | 0.5-250 | 100 | Hz | R |
| P30 | MONITOR SELECT | 0 | 0-1 | 1 | - | R |
| P31 | LINE | 3.0 | 0.1-100 | 10 | Multiple | R/W |
| P32 | MAX.OUTPUT VOLTAGE | 0 | 0-500 | 1 | VAC | R |
| P33 | OCS LEVEL | 140 | 1-200 | 1 | % | R |
| P34 | CARRIER FREQUENCY | 5 | 0-8 | 1 | Code | R |
| P35 | COMMUNICATION STATION | 1 | 1-31 | 1 | - | R |
| P36 | COMMUNICATION SPEED | 5 | 4-6 | 1 | Code | R |
| P37 | STOP BIT | 1 | 1-2 | 1 | Bit | R |
| P38 | PARITY BIT | 0 | 0-2 | 1 | - | R |
| P39 | COMMUNICATION | 0.0 | 0.0-60.0 | 10 | s | R |
| P40 | COMMUNICATION RESPOND TIME | 1 | 1-999 | 1 | ms | R |

| P41 | PASSWORD | 0 | 0-999 | 1 | - | R |
| --- | --- | --- | --- | --- | --- | --- |
| P42 | RESET DATA | 0 | 0-1 | 1 | - | R |
| P43 | SELECT INPUT SIGNAL LOGIC | 27 | 0-31 | 1 | - | R |
| P44 | SAFETY EDGE RESPOND TIME | 10 | 0-999 | 1 | ms | R |
| P45 | POSITION SIGNAL RESPOND TIME | 10 | 1-999 | 1 | ms | R |
| P46 | ENCODE ERROR RESPOND TIME | 0.0 | 0-2.0 | 10 | s | R |
| P47 | PROSITION SIGNAL ERROR RESPOND TIME | 0.0 | 0-10.0 | 10 | s | R |
| P48 | RY1 OUTPUT FUNCTION SELECT | 7 | 0-15 | 1 | - | R |
| P49 | RY2 OUTPUT FUNCTION SELECT | 4 | 0-15 | 1 | - | R |
| P50 | RY3 OUTPUT FUNCTION SELECT | 5 | 0-15 | 1 | - | R |
| P51 | MOTOR PHASE | 6 | 2-6 | 0.5 | Pole | R |
| P52 | ENCODE LINE | ---- | 100-999 | 1 | Pulse | R |
| P53 | SYSTEM PARAMETER (DON’T CHANGE) | 0 | 0-1 | 1 | - | R |
| P54 | SYSTEM PARAMETER (DON’T CHANGE) | 1.0 | 0.1-999 | 10 | Multiple | R/W |
| P55 | SYSTEM PARAMETER (DON’T CHANGE) | 0.0 | 0-999 | 10 | s | R/W |
| P56 | SYSTEM PARAMETER (DON’T CHANGE) | 0.1 | 0.1-999 | 10 | Multiple | R/W |
| P57 | SYSTEM PARAMETER (DON’T CHANGE) | 0.0 | 0-999 | 10 | s | R/W |
| P58 | SYSTEM PARAMETER (DON’T CHANGE)） | 5.00 | 0.5-250 | 100 | Hz | R/W |
| P59 | OVERLOAD FREQUENCY 1 | 10.00 | 0.5-250 | 100 | Hz | R/W |
| P60 | OVERLOAD FREQUENCY 2 | 50.00 | 0.5-250 | 100 | Hz | R/W |
| P61 | OVERLOAD CURRENT 1 | 1.2 | 0.1-100 | 100 | A | R/W |
| P62 | OVERLOAD CURRENT 2 | 1.2 | 0.1-100 | 100 | A | R/W |
| P63 | OVERLOAD RESPOND TIME | 0 | 0-999 | 1 | ms | R/W |
| P64 | OVERLOAD FREQUENCY RATE（LOW SPEED） | 50.00 | 0-100 | 100 | % | R/W |
| P65 | OVERLOAD FREQUENCY RATE（HIGH SPEED） | 70.00 | 0-100 | 100 | % | R/W |
| P66 | OVERLOADSWITCH FREQUENCY | 5.00 | 0.5-250 | 100 | Hz | R/W |
| P67 | OVERLOAD RESPOND TIME | 100 | 0-999 | 1 | ms | R/W |
| P68 | BEGIN RUN RESPOND TIME | 200 | 100-999 | 1 | ms | R/W |
| P69 | FORCE TO OPEN DOOR RESPOND TIME | 0.0 | 0-500 | 10 | s | R/W |
| P70 | ABNORMAL OPEN RESPOND TIME | 0.0 | 0-500 | 10 | s | R/W |
| P71 | ABNORMAL OPEN COMPLETE WAITE TIME | 0.0 | 0-10.0 | 10 | s | R/W |
| P72 | KEEP OPEN TIME (RECYCLE MODE) | 3.0 | 0-10 | 10 | s | R/W |
| P73 | KEEP CLOSE TIME (RECYCLE MODE) | 3.0 | 0-10 | 10 | s | R/W |
| P74 | ACCLE/DECELE CURVE SELECT | 1 | 0-2 | 1 | - | R/W |
| P75 | OPEN/CLOSE STOP MODE SELECT | 0 | 0-1 | 1 | - | R/W |
| D00 | CHECK PARAMETER D PASSWORD | 0 | 0-999 | 1 | - | R/W |
| D01 | DOOR WIDTH PULSE | DEP | 0-65535 | 1 | - | R/W |
| D02 | CLOSE COMPLETE POSITION | 0.00 | 0-100 | 100 | % | R/W |
| D03 | OPEN CHANGE SPEED POSITION 1 | 1.50 | 0-100 | 100 | % | R/W |
| D04 | OPEN CHANGE SPEED POSITION 2 | 16.00 | 0-100 | 100 | % | R/W |
| D05 | OPEN CHANGE SPEED POSITION 3 | 50.00 | 0-100 | 100 | % | R/W |
| D06 | OPEN CHANGE SPEED POSITION 4 | 70.00 | 0-100 | 100 | % | R/W |
| D07 | OPEN CHANGE SPEED POSITION 5 | 80.00 | 0-100 | 100 | % | R/W |
| D08 | OPEN COMPLETE POSITION | 100.00 | 0-100 | 100 | % | R/W |
| D09 | CLOSE COMPLETE POSITION 1 | 95.00 | 0-100 | 100 | % | R/W |
| D10 | CLOSE COMPLETE POSITION 2 | 75.00 | 0-100 | 100 | % | R/W |
| D11 | CLOSE COMPLETE POSITION 3 | 55.00 | 0-100 | 100 | % | R/W |
| D12 | CLOSE COMPLETE POSITION 4 | 15.00 | 0-100 | 100 | % | R/W |
| D13 | CLOSE COMPLETE POSITION 5 | 13.00 | 0-100 | 100 | % | R/W |
| D14 | KEEP CLOSE DOOR FREQUENCY | 2.00 | 0-250 | 100 | Hz | R/W |
| D15 | OPEN DOOR FREQUENCY 1 | 2.00 | 0-250 | 100 | Hz | R/W |
| D16 | OPEN DOOR FREQUENCY 2 | 2.00 | 0-250 | 100 | Hz | R/W |
| D17 | OPEN DOOR FREQUENCY 3 | 23.00 | 0-250 | 100 | Hz | R/W |
| D18 | OPEN DOOR FREQUENCY 4 | 21.00 | 0-250 | 100 | Hz | R/W |
| D19 | OPEN DOOR FREQUENCY 5 | 23.00 | 0-250 | 100 | Hz | R/W |
| D20 | OPEN DOOR FREQUENCY 6 | 3.00 | 0-250 | 100 | Hz | R/W |
| D21 | KEEP OPEN DOOR FREQUENCY | 3.00 | 0-250 | 100 | Hz | R/W |
| D22 | CLOSE DOOR FREQUENCY 1 | 3.00 | 0-250 | 100 | Hz | R/W |
| D23 | CLOSE DOOR FREQUENCY 2 | 3.00 | 0-250 | 100 | Hz | R/W |
| D24 | CLOSE DOOR FREQUENCY 3 | 19.00 | 0-250 | 100 | Hz | R/W |
| D25 | CLOSE DOOR FREQUENCY 4 | 20.00 | 0-250 | 100 | Hz | R/W |
| D26 | CLOSE DOOR FREQUENCY 5 | 19.00 | 0-250 | 100 | Hz | R/W |
| D27 | CLOSE DOOR FREQUENCY 6 | 2.00 | 0-250 | 100 | Hz | R/W |
| D28 | OPEN ACCE/DECE TIME 1 | 0.5 | 0.0-999 | 10 | s | R/W |
| D29 | OPEN ACCE/DECE TIME 2 | 0.5 | 0.0-999 | 10 | s | R/W |
| D30 | OPEN ACCE/DECE TIME 3 | 1.20 | 0.0-999 | 10 | s | R/W |
| D31 | OPEN ACCE/DECE TIME 4 | 0.5 | 0.0-999 | 10 | s | R/W |
| D32 | OPEN ACCE/DECE TIME 5 | 0.5 | 0.0-999 | 10 | s | R/W |
| D33 | OPEN ACCE/DECE TIME 6 | 1.00 | 0.0-999 | 10 | s | R/W |
| D34 | CLOSE ACCE/DECE TIEM 1 | 0.5 | 0.0-999 | 10 | s | R/W |
| D35 | CLOSE ACCE/DECE TIEM 2 | 0.5 | 0.0-999 | 10 | s | R/W |
| D36 | CLOSE ACCE/DECE TIEM 3 | 1.00 | 0.0-999 | 10 | s | R/W |
| D37 | CLOSE ACCE/DECE TIEM 4 | 0.5 | 0.0-999 | 10 | s | R/W |
| D38 | CLOSE ACCE/DECE TIEM 5 | 0.5 | 0.0-999 | 10 | s | R/W |
| D39 | CLOSE ACCE/DECE TIEM 6 | 1.00 | 0.0-999 | 10 | s | R/W |
| D40 | KEEP OPEN DOOR CURRENT | 0.5 | 0-100 | 10 | A | R/W |
| D41 | KEEP CLOSE DOOR CURRENT | 0.4 | 0-100 | 10 | A | R/W |
| D42 | KEEP OPEN/CLOSE DOOR TIME | 0 | 0-999 | 10 | s | R/W |
| D43 | OPEN COMPLETE STAY FREQUENCY | 0.50 | 0.5-250 | 100 | Hz | R/W |
| D44 | CLOSE COMPLETE STAY FREQUENCY | 0.50 | 0.5-250 | 100 | Hz | R/W |
| D45 | OPEN COMPLETE STAY TIME | 0 | 0-10 | 10 | s | R/W |
| D46 | CLOSE COMPLETE STAY TIME | 0 | 0-10 | 10 | s | R/W |
| D47 | BEGIN OPEN DOOR TIME（4SW MODE ） | 0.30 | 0-3 | 100 | s | R/W |
| D48 | BEGIN OPEN DOOR CHANGE SPEED TIME （4SW MODE） | 0.00 | 0-3 | 100 | s | R/W |
| D49 | BEGIN CLOSE DOOR TIME（4SW MODE ） | 0.30 | 0-3 | 100 | s | R/W |
| D50 | BEGIN OPEN DOOR CHANGE SPEED TIME （4SW MODE） | 0.00 | 0-3 | 100 | s | R/W |
| D51 | THE FREQUENCY OF POWER ON | 3.00 | 0.5-250 | 100 | Hz | R/W |
| D52 | DOOR WIDTH AUTOTONE | 2.00 | 0.5-250 | 100 | Hz | R/W |
| D53 | PARAMETER D PASSWORD | 0 | 0-999 | 1 | - | R/W |

9、Troubleshooting

9、1  Surveillant parameter

| NO | NAME | PRECISION | UNIT | REMARK |
| --- | --- | --- | --- | --- |
| n00 | SOFTWARE NO. |  | CODE |  |
| n01 | OUTPUT FREQUENCY | 0.001 | Hz |  |
| n02 | OUTPUT CURRENT | 0.1 | A |  |
| n03 | OUTPUT VOLTAGE | 0.1 | Vac |  |
| n04 | INPUT DC VOLTAGE | 0.1 | Vdc |  |
| n05 | THE SET FREQUENCY | 0.01 | Hz |  |
| n06 | REBACK FREQUENCY | 0.01 | Hz | MUST TO MOTOT PHASE ,ENCODE LINES |
| n07 | DOOR POSITION AREA | 1 |  | 0-7：OPEN DOOR AREA； 8-13：CLOSE DOOR AREA |
| n08 | DOOR WIDTH（PULSE） | 0.01 |  | 0.01=1；100=10000 |
| n09 | DOOR COMPLETE POSITION（DATA） | 0.01 |  |  |
| n10 | DOOR RUN TIMES | 1 | TIME | 0.01=1 TIME；100=10000TIME |
| n11 | ENCODE REBACK STATE | 0.1 | kHz | F：（OPEN）R：（CLOSE） |
| n12 | ERROR 1 |  | CODE |  |
| n13 | ERROR 2 |  |  |  |
| n14 | ERROR 3 |  |  |  |
| n15 | ERROR 4 |  |  |  |
| n16 | INPUT SIGNAL STATE |  |  |  |
| n17 | OUTPUT SIGNAL STATE |  |  |  |

9、2  Error Code

| PANEL SHOW | ERROR CONTENT | REASON | REMEDY | Relevant parameter |
| --- | --- | --- | --- | --- |
| SC1 | Over current or abnormal heating | Output .earth short ambient temperature Is too high Acceleration time is Too short | Check for output short circuits and ground faults Check ambient temperature and fan operation Increase the acceleration time | Acceleration time |
| SC2 |  |  |  |  |
| SC3 |  |  |  |  |
| OC1 | Over current | Output less phase Dc boost level is too High Acceleration is too short | Check for open circuit output phases Increase the acceleration time Adjust the boost level | Acceleration/deceleration time P05 |
| OC2 |  |  |  |  |
| OC3 |  |  |  |  |
| OV1 | Excessive internal DC voltage during deceleration(overvoltage) | Deceleration time is too short | Increase the acceleration time Connect a brake resistor | Acceleration/deceleration time P17 |
| OV2 |  |  |  |  |
| OV3 |  |  |  |  |
| LV | Supply voltage drops to 85% or less of rating (undervoltage) | Voltage supply is low Instantaneous power Cut | Measure the supply voltage Consider the ride-through restart function | P23-25 |
| OL | An output current that is 125% or more of the electronic thermal setting current or 150% or more of the inverter’s rated current continued for one minute or more.(overload) | Actuating of Thermal relay Overload | Check the electronic thermal setting current Reduce the load | P05 P06,P07 |
| AU | An auxiliary fault stop signal was input from control circuit terminal (auxiliary stop) | Emergency stop signal is send | Inspect the sequence circuit to confirm that the auxiliary signal is correct |  |
| OP | The operation panel was input from control circuit terminal (auxiliary stop) | When power is,the run signal is ON When running,the keypad is shell Communication is overtime | Take care with the handing of the operation panel Check the run signal when the data is being set Check the POWER LOSS START MODE | P23 P39 |
| Er1 | Encode no pulse input | Encode is not pulse input | To sure the encode power ,the encode connect | P51,P52 P46 |
| Er2 | Encode phase is wrong | Encode A/B phase is Not right | To sure the encode A/B phase | P51,P52 P46 |
| Er3 | The open/close Complete input are ON | The open/close complete input are ON | To sure the open/close input signal | P43 P45 |
| Er4 | The open complete input is not receive | Door width pulse is show the door open complete ,but the open complete signal is not receive | To sure the open Complete signal | P43 P45 |
| Er5 | The close complete input is not receive | Door width pulse is show the door close complete ,but the close complete signal is not receive | To sure the close Complete signal | P43 P45 |

Annotions:

After the parameter had been setted completely,in rder to prevent change the parameter by accident ,you can set password.

Parameter P41: full section password(include all the parameter of Pand d) to set

The range of password is 1 to 999 (if you set as 000, it means no password)

When you have setted passwoed ,if you want to change the parameter,you should following this:

First press “STOP”key to stop the controller,then press “MODE”key 4 times

If you see word “ps”is glittering ,please press  or  key to input your password

Press “SET” key ,you can change the parameter.

(the controller is no password from factory, if you want to set ,please record password to prevent forget)